BUDDHA SENSOR

To: Jason Loeb

125 Atlantic Street

Quincy, MA 02171

From: Elisha Tam

20800 Boyce Lane

Saratoga, CA 95070

Shipped Supplies

- 1 cardboard container (blow dryer box), containing Arduino, green perfboard, & relay, 5 load cell amplifier boards

  - Be very careful when removing this from the box and bag. Try to pull the cardboard and not the wires.

  - The fuse cable is taped to the bottom of the cardboard mat. You can remove the tape to free the fuse cable

- 1 white box, containing the 12V DC Power Adapter 2.1mm 1A (to power the Arduino)

- 1 cardboard box, containing the 12V DC Power Adapter 2A (to power the magnetic lock) + 1 connector

- 5 scales

  - 4 wires of each scale are taped inside the scale for protection. You can remove the tape.

Demo on Youtube

Component Introduction: https://youtu.be/nYKxuLt1_iM

Weight Puzzle Power Up: https://youtu.be/TLgMqSTYA1w

Weight Puzzle Demo: https://youtu.be/K7c_nsLuTCs

Electrical Setup

- There are a lot of connections involved. Elisha would be happy to walk you through this setup over FaceTime and before you plug any components to power.

- Follow the diagram to connect the 5 scales to their respective Load Cell Amplifier. You should only need to connect

  - The 4 wires of each of the 5 scales

  - The yellow and blue wire of the button

  - The fuse

  - The magnetic lock

    - Connect the red cable to “LCK” on the green board

    - Connect the black cable to the NEGATIVE terminal of the adapter

  - Power to the Arduino

- Connect the adapter to the 2A power plug

- Check all connections are secure

- Make sure nothing is on the 5 scales before you power up the Arduino – the scales tare at power up

- Plug the 2A Power Plug into an outlet (for the magnetic lock)

- Plug the 1A Power Plug into an outlet (for the Arduino).

Software code

- Once the Arduino powers on, wait for the red LED to turn on. The lock will be locked.

- Push the button to set the target weights on the scale. The red LED will now flash every 0.5sec

- Place your 5 objects on the scale.

- Push the button to save the target weights. The red LED will now be OFF

- Remove one or all of the weights to start the game. The red LED will now be ON. The game is ready to play

- Place the correct weights on the scales. The lock will UNLOCK

Costs

|  | Unit Cost | Qty | Total |
| --- | --- | --- | --- |
| Mechanical Components |  |  |  |
| Arduino Uno | 25.81 | 1 | 25.81 |
| SchmartBoard perf board | 5.43 | 1 | 5.43 |
| 12V DC Power Adapter 2.1mm 1A | 5.69 | 1 | 5.69 |
| 12V DC Power Adapter 2.1mm 2A + 2.1mm adapter | 6.22 | 1 | 6.22 |
| Fuse Holder | 2.69 | 1 | 2.69 |
| Nylon washers (x20) | 9.075 | 1 | 9.075 |
| ABS Plastic | 6.26 | 1 | 6.26 |
| M4 Screws | 6.15 | 1 | 6.15 |
| M5 Screws | 7.35 | 1 | 7.35 |
| Magnetic lock (not provided) | 35.99 | 1 | 35.99 |
| Electrical Components |  |  |  |
| 2N7000 N Channel MOSFET | 0.4 | 1 | 0.4 |
| Diode | 0.5 | 1 | 0.5 |
| JS1-B-5V-F Relay | 1.86 | 1 | 1.86 |
| Load cell - 10kg (22lbs), Straight bar (TAL220) | 6.95 | 5 | 34.75 |
| HX711 amplifier | 9.95 | 5 | 49.75 |
| Push button | 1.96 | 1 | 1.96 |
| Fuse (250V, 1A) | 0.95 | 1 | 0.95 |
| Time |  |  |  |
| Took 10 hrs. 1hr to prepare, 6hrs to implement, 3hrs to clean up and document. | 50 | 10 | 500 |
| Shipping |  |  |  |
| UPS 3Day Shipping |  |  | 47.93 |
| Spark Fun shipping |  |  | 5.63 |
|  |  | TOTAL | 754.395 |